# Stepper driver

QJ5045 2-Phase Mircostepping Motor Driver Characteristics: 1.DC power input type:24V~50VDC 2.Output current:1.3A-4.5A 3.Mircostepping:1(1.8º) 1/2 1/4 1/8 1/16 1/32 1/64 1/128 1/256 1/5 1/10 1/25 1/50 1/125 1/250 4.Protect form :Overheated protect, lock automatic half current ,error connect protect 5.Dimensions:118mm×76mm×33mm 6.Weight:<300g. 7.Working environment:Temperature-15～40℃ Humidity<90. I/O Ports: 1.VCC+:DC power positive pole Note:Must guard against exceeding 50V, so as not to damage the module 2.GND:DC power cathode 3.A+ A-:Stepping motor one winding 4.B+ B-:Stepping motor other winding 5.PUL+ PUL -:Stepping pulse input+5V (Rising edge effective, rising edge duration >10μS) 6.DIR + DIR-:Stepping motor direction input, voltage level touched off, high towards, low reverse 7.ENA+ ENA-: motor free NOTE: 1. When ambient temperature is high or working current over 2.7A, fix the module on big metal shell , or use axle flows fan dispels the heat, to make the module run reliably for a long time. 2. Half current automatically: if control machine not send out signal in half second, driver enter half Switch Choice: (“ON=0,OFF=1”) 1.Microstepping choice: SW5; 0; 0; 0; 0; 0; 0; 0; 0; 1; 1; 1; 1; 1; 1. SW6; 0; 1; 0; 1; 0; 1; 0; 1; 0; 1; 0; 1; 0; 1. SW7; 0; 0; 1; 1; 0; 0; 1; 1; 0; 0; 1; 1; 0; 0. SW8; 0; 0; 0; 0; 1; 1; 1; 1; 0; 0; 0; 0; 1; 1. Micro; 1/2; 1/4; 1/8; 1/16; 1/32; 1/64; 1/128; 1/256; 1/5; 1/10; 1/25; 1/50; 1/125; 1/250. 2.Current choice: SW1; 0; 1; 0; 1; 0; 1; 0; 1. SW2; 0; 0; 1; 1; 0; 0; 1; 1. SW3; 0; 0; 0; 0; 1; 1; 1; 1. Current（A）; 1.3; 1.8; 2.3; 2.7; 3.2; 3.7; 4.2; 4.5. 3.Full current or half current choice: SW4:0＝Full current,1＝half current 4.Pulse choice: In the driver has a jump 5.installation dimension