M6Start.mis

Note that I move the A axis 72 degrees, then back up 12. This should cause the ratchet to lock up. Also note all A axis moves are made in relative mode (G91).

*******************************

Sub main

tool=getcurrenttool()

'MsgBox ("current tool " & tool)

NewTool= getselectedtool()

'msgbox ("new tool " & NewTool)

if NewTool=tool then

  exit sub

end if

if NewTool>tool then

   steps=Newtool-tool

else

   steps=6 - (tool-NewTool)

End If

If tool Mod 2 Then 'even number

   code "G00 Z 2"

Else

   code "G53 G00 X 0"   'move X to machine home 0

   code "G00 z -2"

End If

'msgbox(" move steps " & steps)

for I=1 to steps

   code "G91 G00 A72"

code "g00 A-12"

code "G90"

While IsMoving()

Wend

next I

end sub

main

