This will dwell for 3 seconds.

 CurrentFeed = GetOemDRO(818) 'Get the current feedrate.
PlateThickness =0.375

If GetOemLed (825)=0 Then
DoOEMButton (1010)
Code "G4 P3" 'Time to get to the z-plate, 3 seconds
Code "G90 G31 Z-2 F6" ' probing move, can set the feed rate here as well as how far to move
While IsMoving()
Wend
ZProbePos = GetVar(2002)
Code "G1 Z" &ZProbePos
While IsMoving ()
Wend
Call SetDro (2, PlateThickness)
Sleep(120)
Code "G0 Z2" 'Change the Z retract height here to what you want, must be higher than touch plate
Code "(Z axis is now zeroed)"
Code "F" &CurrentFeed 'returns to prior feedrate
Else
Code "(Z-Plate is grounded, check connection and try again)"
End If 
Exit Sub

