Rem VBScript To center probe Inside Diameter Metric Version

CurrentFeed = GetOemDRO(818) 'Get the current feedrate to return to later

CurrentAbsInc = GetOemLED(48) 'Get the current G90/G91 state

CurrentGmode = GetOemDRO(819) 'Get the current G0/G1 state

CurrentToolDiameter = GetOemDRO(43) 'gets the current tool diameter

If GetOemLed (825) <> 0 Then 'Check to see if the probe is already grounded or faulty

Call ProbeGrounded()

Exit Sub

Else

XCurrent = GetDro(0)

YCurrent = GetDro(1)

Code "G4 P1" 'Pause 1 second to give time to position probe plate

Code "F100" 'slow feed rate to 100mm/sec

Rem Probe Left

XNew = Xcurrent - 75 'probe 75mm to left

Code "G31 X" &XNew

While IsMoving() 'wait for the move to finish

Wend

XPos1 = GetVar(2000) 'get the probe touch location

Code "G0 X" &XCurrent 'rapid move back to start point

Rem Probe Right

XNew = XCurrent + 75 'probe 75mm to right

Code "G31 X" &XNew

While IsMoving()

Wend

XPos2 = GetVar(2000)

XCenter = (XPos1 + XPos2) / 2 'center is midway between XPos1 and XPos2

Code "G0 X" &XCenter 'rapid move to the x center location

While IsMoving ()

Wend

Call SetDro (0,0.000)

Code "G4 P0.25"

Rem Probe up

YNew = YCurrent + 75

Code "G31 Y" &YNew

While IsMoving()

Wend

YPos1 = GetVar(2001)

Code "G0 Y" &YCurrent

Rem Probe down

YNew = YCurrent - 75

Code "G31 Y" &YNew

While IsMoving()

Wend

YPos2 = GetVar(2001)

YCenter = (YPos1 + YPos2) / 2

Rem move To the center

Code "G0 Y" &YCenter

While IsMoving ()

Wend

Call SetDro (1,0.000)

While IsMoving ()

Wend

Call SetUserDro (1152,YPos1 - YPos2 + CurrentToolDiameter)

Code "G4 P0.25"

Code "F" &CurrentFeed 'restore starting feed rate

Call ReturnG90G91State()

Exit Sub

End If

Sub ProbeGrounded()

Code "(Probe plate is grounded, check connection and try again)"

Call ReturnG90G91State()

End Sub

Sub ReturnG90G91State()

If CurrentAbsInc = 0 Then 'if G91 was in effect before then return to it

Code "G91"

End If

If CurrentGMode = 0 Then 'if G0 was in effect before then return to it

Code "G0"

End If

End Sub