CNC Start up and Job run

Start up:

Clean & lube		Jog all axis whilst lubing system

Lift Z up to clear obstacles and clear table

Reference all home	Program run

Hang up dust collector

Ref’ all home, (red)

Go to user (black)

Zero X & Y

Check machine coordinates at zero

G54	Machine zero point

G55	Job zero point

Offsets	G54 – G59	various X – Y pre settings.

Run a job:	Load table with material and first tool bit.

Open folder- found in, (example lift v37).

Load a job	Load G code, (example main comp’/main HDD/

CAM/Lift door)

Loads codes, (check for G54/G55)

Offsets – change to G_ _ as set in the code.

Tool path will bring up larger display

Purple current center of tool, red is the job tool

path, blue is the stock, dotted white is the

table.

Check program limits within material.

Check the code, (G55 or 54, tool change what tool, depth of job for holes/ tracks, feeds & speeds).

Check the job process	Push Offline,

Start – follow tool path on screen.

Stop, rewind. – done!

Check part of the program	Push offline.

Go to new starting point.

Push run from here, (will check this part, and return to this start point).

Check screen – OK, cycle start.

Cycle start to run, (will show on screen).

Return to a code in the program	Code number/enter, set next line.

This can be used when checking the code or in

real time such as a broken tool bit to start

again from this point.

Moving tool to job zero point	MDI

GO X___Y___ input, (full speed).

G1 X ___Y ___input, (jog speed).

Offsets

Fixtures, check G55, (current job zero point)

Zero X & Y- esc out

Program run		Tool zero	Jog Z close

Plate and contact on

Auto tool zero

Tip- ready on ESC to stop!

Regen tool path

Rewind, (returns to start of program)

Start job, (runs to job until tool change)

Want to hold the job, (feed hold)

Restart job, (feed hold)

Changed the tool, (start)

Stop, (stop)

Jogging 		Tab, (jog speed and mode)

Continuous, (arrows)/ Step, (set step)

Ctrl J – set/ change step rate for setting up XY

Jog speed, (change% rate or entry/ enter)

Shift/ arrows, (top speed)