Open Mach 4

Select Operator > Edit Screen

Click on the “Add a Lua Panel” icon

On the left side of the screen, in the tree, find the new Lua file and click on it

In the “Properties” box below, rename your Lua file something describing the script you are adding

Also click the “Hidden” box or you might have problems accessing your screens

In the “Properties” box, click the icon with the lightning bolt

Click inside the “Script” box, and then click on the ellipsis on the right side of the box

A new blank screen will appear.  Write or paste your script into this screen.

When you are done, save your script (File > Save or Save icon)

Click the “X” in the top right corner to exit the screen

Select Operator > Edit Screen

When prompted to “save the current screen”, click “Yes”

If errors are detected in your code, you will be prompted with messages about the errors

If you are testing the jogging functions of an Xbox 360 controller, be sure to press the flashing green “Enable” button before testing.  Press the red “Disable” button when you are done testing.