Folks,
I am hoping for some help as I am scratching my head with this error when Homing.”Requested home axis home switch is active. Please fix, then home”

Here is a summary of machine specs
I have upgraded an oldish hobby cnc machine (ooznest workbee) with a mach 3 card.... the … BSMCE04U (image attached, it’s the red square one)

My configuration is 4 stepper motors with torque of 1.26NM, 200 steps per rev. Lead screw is 8mm 4 start, drivers are set to 8 micro steps so steps per mm is 200 steps per mm.

X Axis

Y axis with A axis slaved to it

Z axis

I have been able to get the motors to turn and I have been able to get the axes to move (jog) in the directions that is correct for positive and negative.

Standing at the front of the machine, I consider machine 0,0 to the near left corner. So technically the machine is homing to the back right corner.

Limit Switches
I have my home and limit switches configures also. I have at present 3 limit switches which also function as homing switches. These are at the positive max of the three axes (I.e. the back right corner of the machine).

These witches are Normally Open and they are connected in parrallel to the card

When I trip the switches on the diagnostics screen the screen lights up when the switch is pressed and is dark when not pressed.

All switches work correctly and were tested.

The Problem
I have been trying to get my machine to home and the following happens

The z axis successfully moves up to find home and successfully homes

The y axis give the error above, but still travels to find the limit switch., it finds the limit switch and then nothing.

The x axis does not try to home.

The x axis does jog as expected.

I have replaced all limit switches and wires.

I have checked all connections 

I have changed the 

Settings I have tried to see what effect they have
Changed Input Signals Active Low on the Homing.

Changed the A Axis Emulated to Tick (yes)

Changes the Home/ Soft Limits Reversed Flag

Switched wiring connections to different places and different connectors.

It always endsup the y axis throws the error above

Here is the Motor Configuration

[image: image1.png]Engine Configuration... Ports & Pins

PotSctp and i Seecion  Notor O |t Sgnas | Ot Signds | Encode/MPG | Spinde Sctup | 1 Optns |

Signal Ensbled |StepPin® | DirPin: StepLowA...StepPort | DirPort
XAxis < o o < < o o
¥ Axis < o o x < o o
Zadis < o o < < o o
amis |4 o o x < o o
B Axis x 0 0 x x 0 0
C Axis x 0 0 x x 0 0
Spindle x 1 0 x x 3 0

Apply





Here is the inputs Configuration 

[image: image2.png]Engine Configuration... Ports & Pins

Pot Setup and s Selecton | itor Ot

Input Signais | Output Signals | Encoder/MPG's | Spindie Setup | Mil Options |

Signal Enabled Port# Pin Number | ActiveLow |Emulated | HotKey
Xes L4 3 2 x x 0
b= 1 4 3 2 1 4 1 4 o
X Home L4 3 2 x x 0
Vs L4 3 2 x x 0
1 4 3 2 1 4 1 4 o
L4 3 2 x x 0
Z+s L4 3 2 x x 0
2= 1 4 3 2 1 4 1 4 o
ZHome L4 3 2 x x 0

Fins 10-13and 15 are nputs. Oriy these 5 pin numbers may be used on tis screen

Automated Setup of Inputs

o [ caed |

Apply




[image: image3.png]Engine Configuration... Ports & Pins X

PotSckp and i Seecion | Motar s FPut S9ns | Guiput Signds | Enode/MPG | Spinde Sctup | i Optins |

Signal Enabled | Port® Pin Number
A

A
Atome

3
i
H

Emulated | HotKey ~

Br
5
BHome
Cor

X KN K KA KA
ELLLLLLL
ELLLLLLL

CHome

Fins 10-13and 15 are nputs. Oriy these 5 pin numbers may be used on tis screen

Automated Setup of Inputs

o [ [




Here is the homing limits configuration

[image: image4.png]Motor Home/SoftLimits

e re nsehip .
o [Revsed [5ofMax_[SoRtMin. | lowZone | Home OF. Home .. | Ao Zero [Spesd %
X (¥ ww oo e ome ¥ @
V€ we oo e ome ¥ o @
7 X ee oo e e ¥ o @
A€ me oo e e ¥ o @
5 [0 w0 e e ome ¥ @
C ™ mw oo e ome ¥ o @
 28hemeocatincordntes——
x NG
v s o
: <

=1





Here is the slave axis configuration

[image: image5.png]Slave Axis Selection

Riestat Mach3 afte resetting these selections

X s ¥ sis 2 s
Slaved Aris Slaved Avis Slaved Avis
© Abis & Adis © Adis
© Bhvis © Bhvis © Bavis
© Chvis © Chvis © Casis
& Nore € None  Nore

oK





Here are the Y axis (Closest) and the A Axis (far away axis) the wiring is messing as I am trying to get it to work before I tidy all cables up. Limit switch is on the near axis (Y axis)

[image: image6.jpg]



Any Help would be greatly appreciated, I think it’s a software issue (My Mach 3 is demo version)  some configuration somewhere.

The PC is a toshiba laptop with an i7 processor and 16GB Ram

